| Task/Equipment: | Reassembling Winter Boiler After Annual Inspection |
| --- | --- |
| Department: | Powerhouse |
| Analyzed By: | Bryana Borders & Bill Partin |
| Date: | 4/12/19 |

Job Hazard Analysis

Personal Protective Equipment Required:

Heavy Leather Gloves

Hard Hat

Trainings Required*/Recommended:

Proper ladder use training*, OSHA 10

Equipment/Tools/Chemicals Required for the Job:

Pipe wrenches, socket wrenches, screw drivers, hand wrenches, sanding cloth, hammer, rope, flashlight, flood light

| Tasks/Steps | Hazards Present | Safety Measures and Controls |
| --- | --- | --- |
| - Reinstall manway covers with new gaskets and discard old gaskets (hammer may be used) | Finger pinch and finger/hand smash from hammer | Use heavy leather gloves to protect from pinching. Keep hands away from the area where blunt force is applied |
| - Replace all cross tees and inspection plugs | Fall from ladder | Follow proper ladder use protocol |
| - Replace gasket and reinstall water level safety float and secure bolts | No Hazards Present | N/A |
| - Check that the bottom blowdown valves are closed and remove lock-out device from valves | No Hazards Present | N/A |
| - Close fire-side access door, replace bolts and secure | Wrist twist | Operate impact wrench carefully and use heavy leather gloves for grip |
| - Remove lock-out device from main power disconnect and close | No Hazards Present | N/A |
| - Close main boiler gas cock and leave handle in the vertical position | No Hazards Present | N/A |
| - Re-open OS&Y valve on top of boiler, leave non-return closed until boiler is put online | Ladder fall | Follow proper ladder use protocol |
| - Re-enable chemical feed pump if applicable | No Hazards Present | N/A |
| - Reopen surface blowdown stop valve closest to the boiler and remove tag | No Hazards Present | N/A |

Additional Information:

Protection Suit

Fall Protection Harness

High Vis Vest

Safety Shoes

Splash Goggles

Latex Gloves

Dust Mask

Respirator

Face Shield

Safety Glasses

Ear Protection