Job Hazard Analysis

| Task/Equipment: | Chemical testing and handling |
| --- | --- |
| Department: | Powerhouse |
| Analyzed By: | Bill Partin & Bryana Borders |
| Date: | 5/3/19 |

Personal Protective Equipment Required:

Face Shield

Latex Gloves

Dust Mask

Insert Picture of Task

Safety Glasses

Trainings Required*/Recommended:

Safe chemical handling

Equipment/Tools/Chemicals Required for the Job:

Dolly

| Tasks | Hazards Present | Safety Measures and Controls |
| --- | --- | --- |
| Adding eternal boiler treatment/condensate treatment chemical to feed tanks | Chemical splash eye/skin contact | Use full face shield, extra-long rubber gloves and long sleeve shirt; use dolly to transport heavy chemical barrels |
| Adding oxygen scavenger powder ( sulfite) to DA feed tank | Inhalation on powdered chemical, contact with skin and eyes; back strain from moving oxygen scavenger powder | Use full face shield, rubber gloves, particulate respirator; use dolly to transport heavy chemical barrels |
| Adding salt to brine tank | Back and muscle strain due to lifting salt bags 40-50 lbs. | Ask for assistance when lifting bags |
| Chemical plant testing/titration | Chemical to skin and eye contact | Use rubber gloves and safety glasses |

Additional Information:

Lab Coat

Protection Suit

Fall Protection Harness

High Vis Vest

Safety Shoes

Splash Goggles

Cryogenic Gloves

Respirator

Hard Hat

Safety Glasses

Ear Protection